AUTONOMOUS ROBOT CAR

# Obstacle Navigation
System Architecture

v0.5.10  |  Simulation / Physical Parity Review

Android phone + Arduino UNO + L298N + HC-SR04

| PERCEPTION CameraX + free-space ONNX + traffic-sign TFLite | PLANNING Ground-plane projection, 19 cm effective gap rule |
| --- | --- |
| CONTROL Shared stabilizer, bounded scan state machine | ACTUATION USB serial, Arduino safety veto, L298N motor drive |

Current validation boundary The model and command/state logic have been reviewed; 58 targeted JVM unit methods passed and the Arduino sketch passed C++ syntax checking with API stubs. A full Android Gradle build and a physical chassis acceptance test were not run. Speed and wheel-track parameters are still uncalibrated placeholders.

Purpose: describe the current v0.5.10 implementation, the changes made to narrow virtual/physical differences, and the calibration still required before autonomous floor testing.

# 1. End-to-end system architecture

One behavior authority in Android; deterministic motor output and a last-resort safety veto on Arduino.

| Stage | Input → output | Responsibility |
| --- | --- | --- |
| 1. Camera capture | CameraX frame → upright bitmap | Keeps camera frame processing current; stale frames inhibit motion. |
| 2. Free-space model | 256 × 256 RGB → semantic classes 0 / 1 / 2 / 255 | Class 0 is free; class 1 obstacle; class 2 unknown; 255 ignore. Unknown is never drivable. |
| 3. Traffic-sign model | 640 × 640 RGB → five-class detections | STOP / NO_ENTRY are confirmation-filtered and safety-prioritized. |
| 4. Geometry | Semantic mask → approximate ground-plane coordinates | Camera height, downward tilt and FOV convert pixels into metric clearance estimates. |
| 5. Planner/controller | Tracked mask + sonar + signs → command candidate | Checks full body trajectory, boundary policy, speed and safety gates. |
| 6. Recovery + stabilizer | Blocked-path state → bounded command | Shared ActivePathDiscovery and LiveDecisionStabilizer for live and virtual control. |
| 7. USB serial | COMMAND,SPEED | Serialized command writes, distance/status queries and telemetry. |
| 8. Arduino + L298N | Validated command → two motor PWM outputs | Independent sonar stop, timed scan/backoff pulses and command watchdog. |

## Behavior architecture

PERCEPTION + SENSOR SNAPSHOT

↓

OBSTACLE PLAN → SPEED CONTROLLER

↓

ACTIVE PATH DISCOVERY (only on persistent blocked path)

↓

LIVE DECISION STABILIZER → SERIAL COMMAND

↓

ARDUINO SAFETY CHECK → L298N / MOTORS

## Important ownership boundaries

Android proposes movements and maintains the active navigation state. It does not directly write motor pins.

Arduino has final authority over close-range forward/pivot vetoes and timed scan/backoff pulse duration.

Virtual mode disables USB motor output; it exercises the same control logic against a synthetic scene and simulated sonar.

# 2. Geometry, free space and route selection

The effective gap threshold is 19 cm in both virtual and live planning; the nominal boundary is a separate concept.

| Parameter | Current value | Meaning |
| --- | --- | --- |
| Robot width × length | 15.0 × 25.5 cm | Physical body dimensions; verify protrusions and wheel envelope. |
| Lateral safety margin | 2 cm per side | Adds 4 cm total to the 15 cm base gap requirement. |
| Effective required gap | 19 cm | The value used by both app planner instances and reported in the HUD. |
| Nominal center offset | ±10 cm | Preferred corridor before crossing boundary. |
| Expanded center offset | ±30 cm | Temporary extension beyond nominal; only considered when nominal route is infeasible. |
| Camera pose defaults | Height 10 cm; tilt 7.5° | Initial mounting assumptions; calibrate on the actual phone mount. |
| Camera FOV defaults | Horizontal 62°; vertical 48° | May vary by selected phone camera and resolution. |
| Planning range | 20–90 cm; target lookahead 75 cm | Geometry is approximate and depends on ground-plane calibration. |
| Raster tolerance | 0.5 cm | Handles discretization; not physical safety margin. |

## Planning rules

Only explicitly FREE pixels count as drivable. UNKNOWN and IGNORE pixels cannot establish a verified passage.

Candidate paths are checked over projected depth rows against the effective robot footprint, not just a single center pixel.

The planner prefers a clear central route, then a nominal-boundary avoidance route, then an expanded-boundary route; if no route is feasible, it returns STOP.

The green overlay means a currently feasible/authorized candidate exists. Scan direction cues are amber and explicitly unverified; STOP/no-route must not be drawn as a green path.

Telemetry fix in v0.5.10 The live and virtual HUD now show “REQUIRED GAP: 19 cm” under the current 2 cm per-side margin. Earlier versions could show 15 cm even though the planner required 19 cm, which made debugging misleading.

# 3. Control states and safety thresholds

Motion changes require confirmation; safety stops remain immediate. Active recovery never assumes an unverified route is safe.

| Control setting | Value | Purpose / notes |
| --- | --- | --- |
| Controller tick | 100 ms nominal | 10 Hz control update; host scheduling can jitter. |
| Command stabilization | 2 consistent ticks | New translating direction waits for confirmation; STOP and speed decreases are immediate. |
| PWM ceiling | 120 | Hard cap in the Android controller and Arduino sketch. |
| Motor floor | 60 PWM | Placeholder for minimum useful movement under load; calibrate. |
| Slight steering | 45% inner-wheel request; outer minimum 80 PWM | Avoids both wheels collapsing to the same 60 PWM floor. |
| Startup kick | PWM 90 for 60 ms only when top request <80 | Scan pivots at PWM 80 do not trigger this kick. |
| App translation stop | Below 30 cm | Blocks forward and translating commands based on fresh sonar. |
| App pivot gate | Sonar ≥24 cm and camera nearest obstacle ≥22 cm | If sonar reports NO_ECHO, camera clearance must be at least 25 cm. |
| Arduino independent cutoffs | Forward 28 cm; pivot 20 cm | Backstop threshold; current sonar is rechecked before movement. |
| Ultrasonic sample age | Maximum 800 ms | Missing/stale values do not count as clear. |
| Arduino command watchdog | 400 ms | Stops motion if accepted movement commands stop arriving. |

## Safety ordering

STOP, stale perception, user stop and critical sign decisions override path-following and command stabilization.

Normal obstacle stops do not automatically latch the app into SAFE; protocol/watchdog failures still require a fresh safe reading and manual re-arm.

If the Arduino rejects a scan pivot on its current sonar measurement, v0.5.10 aborts the active scan rather than assuming the robot rotated.

The current robot has no wheel encoders or IMU; all physical distance/heading change remains open-loop and is re-estimated from camera views.

Do not remove gates to make the virtual result look better. Calibrate the camera, sonar and motor model first. If required clearance cannot be verified, the controller should stop rather than guess a route.

# 4. Bounded recovery: reverse, scan, re-plan

Recovery is finite and checks clearances between pulses. It does not rotate continuously until a route appears.

| State / transition | Behavior | Exit condition |
| --- | --- | --- |
| IDLE → BACKOFF | After 3 persistent blocked-path frames, with fresh vision/obstacle and numeric front sonar, issue `BACKOFF,80`. | One reverse request; rear space must be known clear by operator. |
| BACKOFF | Arduino runs one PWM-80 reverse pulse for 250 ms. Android keeps request idempotent; virtual distance uses modelled PWM × duration. | After a guard interval covering the firmware pulse, proceed or wait stopped. |
| WAIT_CLEARANCE | No wheel motion. Re-check front sonar and camera-obstacle clearance. | Only when sonar/camera gates pass does the scan start. |
| SCAN LEFT | Up to 4 firmware-timed `SCAN_L,80` pivots, one shot per pulse. | Arduino stops each pulse after 80 ms; app advances at/after next 100 ms tick. |
| SETTLE / SAMPLE | STOP, then wait 200 ms for a new view/obstacle map. | Re-plan and validate clearance before the next pivot. |
| SCAN RIGHT | Up to 8 firmware-timed `SCAN_R,80` pivots; traverses back across the center. | Same clearance check between each pulse. |
| NO_ROUTE / SAFETY_HOLD | Remain stopped if no path appears or a firmware pivot is rejected. | Requires a fresh safe path observation before normal following can take over. |

## Why the firmware now owns the scan pulse timer

In v0.5.9 the app sent normal `L`/`R` commands and expected its next 100 ms control tick to issue STOP. Android scheduling or blocking sonar reads could extend a physical pulse while the simulator always rotated for exactly one frame. v0.5.10 uses idempotent `SCAN_L`/`SCAN_R` commands; the Arduino stops them after 80 ms even if the app’s STOP arrives late.

## Backoff is not an exact 5 cm command

The reverse pulse is 250 ms at PWM 80. The virtual model predicts displacement from the configured speed estimate instead of forcing a 5 cm move. At the default placeholder (18 cm/s at PWM 150), it predicts 2.4 cm. The real distance must be measured on this chassis; the front sensor cannot see behind the robot.

# 5. Hardware, wiring and serial protocol

Arduino UNO executes the motor command and provides independent close-range and timeout checks.

| Arduino UNO pin | Signal | Connection / note |
| --- | --- | --- |
| D2 | IN1 | L298N left motor direction |
| D3 | IN2 | L298N left motor direction |
| D4 | IN3 | L298N right motor direction |
| D5 | IN4 | L298N right motor direction |
| D9 (PWM) | ENA | L298N left motor enable; remove ENA jumper when driven by PWM |
| D10 (PWM) | ENB | L298N right motor enable; remove ENB jumper when driven by PWM |
| D6 | TRIG | HC-SR04 trigger |
| D7 | ECHO | HC-SR04 echo; verify level compatibility/wiring for the target board |
| GND | Common ground | UNO, L298N supply/logic and sensor must share ground |

## Android → Arduino command examples

| Command | Meaning | Firmware behavior |
| --- | --- | --- |
| `F,120` | Forward | Forward gate; speed hard capped at 120 PWM. |
| `SL,80` / `SR,80` | Slight steering | Forward safety gate; 45% inner ratio and outer minimum 80. |
| `L,80` / `R,80` | General pivot | Legacy/manual held pivot; front sonar cutoff at 20 cm. |
| `BACKOFF,80` | One-shot reverse | 250 ms at PWM ≤80; idempotent while active; no rear sensor. |
| `SCAN_L,80` / `SCAN_R,80` | Bounded scan pivot | One-shot 80 ms command; repeated writes do not restart it. |
| `S,0` | Stop | Cancels timed motion and clears one-shot latches. |
| `DISTANCE` / `STATUS` | Telemetry requests | Returns sonar reading or `NO_ECHO`, and readiness status. |

## Serial/firmware notes

USB serial is 115200 baud, 8-N-1, newline-terminated ASCII commands. The Android protocol allowlist and Arduino parser must be updated together.

# 6. What still differs between virtual and reality

The v0.5.10 model is more representative, but synthetic perception and unmeasured physical movement remain major differences.

| Aspect | Virtual harness | Physical robot |
| --- | --- | --- |
| Obstacle perception | Synthetic known world → semantic mask; no model prediction error. | ONNX model predictions → projected geometry; misses, false positives and calibration error possible. |
| Sonar | Deterministic geometric beam from the front-edge sensor position; ±0.5 cm patterned noise. | HC-SR04 may miss soft, angled, narrow or off-axis surfaces; synchronous safety pings can delay code. |
| Motion | Differential-drive equations, PWM floor, steering ratio, startup-kick model, calibrated speed/track parameters. | DC motors, L298N losses, gear mismatch, battery changes, slip, floor friction and coast. |
| Pose | Mathematically integrated from commands. | No encoders/IMU; the app cannot know actual heading or distance between camera observations. |
| Timing | Virtual clock ticks every 100 ms; 80 ms pivots and 250 ms backoff represented explicitly. | Android/USB jitter; firmware owns timed pivot/backoff shutdown. Sonar reads can add latency. |
| Gap safety | 19 cm effective gap rule; swept footprint checks. | 19 cm estimated clear gap plus any unmodelled protrusions/measurement error; real safety margin may need increase. |

## Calibration constants are not measured facts

| Parameter | Current default | What to measure |
| --- | --- | --- |
| Nominal loaded speed at PWM 150 | 18 cm/s | Straight-line travel under load at PWM 60, 80, 100, 120. |
| Wheel track width | 13.5 cm | Distance between left/right wheel contact-center lines; validate yaw with pivot tests. |
| Minimum moving PWM | 60 | Lowest PWM where both wheels reliably turn under chassis load. |
| Camera height / tilt | 10 cm / 7.5° | Actual phone mount geometry and projected distances at several depths. |
| Backoff duration | 250 ms @ PWM 80 | Actual reverse distance over three trials on a clear rearward path. |

Expected limitation No parameter-only change can make ground-truth simulated perception equal a live neural model. If parity remains poor after motor/sensor calibration, the next larger improvement is a replay mode that runs the same ONNX/TFLite models on recorded real camera frames while replaying the matching motor/sonar telemetry.

# 7. Calibration and validation checklist

Run in a clear, bounded area, at low PWM, with an emergency STOP reachable. Do not calibrate near people, pets, stairs or traffic.

| Step | Action | Pass criterion |
| --- | --- | --- |
| 1. Upload consistency | Use the v0.5.10 Android project and its matching `.ino`; do not mix older firmware with `SCAN_L`/`SCAN_R` app commands. | `ROBOT_READY`/`STATUS=OK`; new scan commands are accepted. |
| 2. Motor polarity | Lift wheels and issue low-speed F/REV/L/R/SL/SR. Verify directions. | Physical directions match the labels; use `INVERT_LEFT_MOTOR` / `INVERT_RIGHT_MOTOR` only if required. |
| 3. PWM-speed curve | Mark 50–100 cm floor segment; run PWM 60/80/100/120 three times each. | Use loaded average speeds in virtual model; note unequal left/right speed if present. |
| 4. Scan-pivot timing | Test `SCAN_L,80` and `SCAN_R,80` with rear/front clear. | Direction correct, pulse ends automatically around 80–105 ms; a delayed Android STOP does not extend it. |
| 5. Sonar check | Compare live readings against a ruler at 20/24/30/40/60 cm; include angled/off-center obstacles. | Reading error characterized; if poor, fix sensor aim/threshold margin before autonomous run. |
| 6. Camera projection | Verify phone height, tilt and FOV; compare nearest obstacle estimates with measured distances. | Metric estimates conservative in the 20–90 cm planning band. |
| 7. Backoff distance | With the rear known clear, measure the PWM-80 250 ms reverse pulse three times. | Update model speed or duration so predicted displacement matches measured travel. |
| 8. Obstacle scenarios | Test open straight, central object, wide side route, wall/dead-end, and stale/missing sonar. | Stop on unverified routes; no forward drive through obstacles; no endless scan/oscillation. |

## Validation completed for source package

58 targeted navigation, active-path, stability, model, and serial-protocol JVM test methods passed under a local runner. The runner used a small JUnit-compatible stub; this was not a Gradle/JUnit build.

Arduino `.ino` passed `g++ -fsyntax-only` with Arduino API stubs. This checks syntax, not board-specific upload/runtime behavior.

Full `./gradlew test --no-daemon` was blocked: the project wrapper requires Gradle 9.6+ and the available environment does not provide that distribution / Android build toolchain.

No physical chassis acceptance test has been performed. Physical behavior must be validated after upload and calibration.